CTP PLANNER

Building + Curing Synchronized Scheduling Engine

Complete Technical Explainer

PCR  &  TBR  Plants   |   Reference Month: August 2026

| Document | Detail |
| --- | --- |
| Purpose | How the CTP Planner works - architecture, layers, inputs, flow, results |
| Scope | End-to-end engine behaviour for a single planning month |
| Engine | planner/cmbc/ (live engine) - driven by main.py |
| Reference run | runs/FINAL_aug (month = 2026-08) |
| Data basis | All figures derived from actual run artefacts (parquet files) |

# 0.  Executive Summary

The CTP Planner is a Python-based scheduling engine that produces a full-month synchronized Building and Curing schedule for two plants - PCR (Passenger Car Radial) and TBR (Truck Bus Radial). It takes the plant demand workbook and master files as input, and produces the BTP pack (Excel) and a 13-sheet shift schedule as output.

The single most important design fact is that this plant is CURING-FIRST:

Curing proposes, Building disposes The curing campaign is placed first - which press runs which GT, and between which times. Building is then released BACKWARDS from it, at t_cure minus tau* minus build_duration. Building is not the leader; it is the follower. Green Tyre (GT) has only a 72-hour shelf life (rule R5), so GT is a SYNCHRONIZATION BUFFER, not a production target. Building more GT than curing can consume simply produces tyres that expire.

August 2026 actual result (run: FINAL_aug):

| Metric | PCR | TBR | Comment |
| --- | --- | --- | --- |
| Demand (tyres) | 4,29,146 | 99,019 | Plant order book |
| Cure campaigns placed by L5 | 4,12,897 | 98,625 | Limited by press capacity |
| BUILT (actual production) | 4,08,950 | 97,985 | This is the real production figure |
| Cured in-month (fed) | 3,84,464 | 92,983 | Entered a press within the month |
| Build machine utilisation | 83.5 % | 81.7 % | Roughly 17-18 % capacity idle |
| Press utilisation | 102.6 % | 100.4 % | Running past the month boundary |
| Build capacity ceiling | 5,02,839 (117 %) | 1,17,572 (119 %) | Building is NOT the bottleneck |
| Cure capacity ceiling (modelled) | 4,07,396 (95 %) | 1,06,415 (107 %) | PCR is constrained here |

Three-line summary:

PCR production is already above 4 lakh (4,08,950 built). What is not being met is the full demand of 4,29,146.

Building is not the bottleneck - it has roughly 17 % spare capacity. The binding constraint is CURING.

The dip seen at the start and end of the month accounts for almost the entire PCR shortfall (dip loss ~16,346 tyres against a total gap of 16,249 tyres). Section 7 gives the full calculation.

# 1.  What the CTP Planner Is - Problem Statement

## 1.1  The problem being solved

A tyre plant has two coupled production stages:

BUILDING (TBM machines) - produces the Green Tyre (GT), an uncured tyre.

CURING (presses) - applies heat and pressure to the GT to produce the finished tyre.

GT inventory sits between them. The difficulty is that a GT can only be used within 72 hours. Build too much and it expires; build too little and presses stand idle. Both are costly.

The engine must therefore balance five objectives simultaneously:

Meet demand (fulfilment).

Ensure GT inventory is consumed within 72 hours (rule R5).

Respect the physical limits of moulds, presses and machines (R3, capacity).

Minimise changeovers (mould changes, size changes) - each one consumes capacity.

Keep lot sizes within plant rules (B12 floor: PCR 150, TBR 70).

## 1.2  Technical stack, and the reasoning behind it

| Choice | What is used | Why |
| --- | --- | --- |
| Language | Python 3.11 | Maintainable by the plant team; strongest data libraries |
| Data engine | Polars + DuckDB + Parquet | Pandas stalls on 4.4 GB of MES data. Polars is 10-50x faster |
| Algorithm | Heuristic greedy + SA / Tabu / LNS | CP-SAT / MILP deliberately excluded - measured, no benefit found |
| Learning | Classical statistics + pattern mining | No ML / RL / LLM - the plant requires explainable output, not a black box |
| Storage | Every layer communicates only through files | Any single layer can be re-run in isolation; every number is traceable |

These are deliberate decisions, not limitations Both ML and MILP were trialled and rejected with measurement. The L9 optimiser was given a 300-second budget: it searched 1,428 candidates and accepted 0 moves, leaving all nine cost tiers unchanged. It was therefore removed. This is not "we could not do it" - it is "we measured it and it did not help".

## 1.3  Repository structure

| Folder | Contents |
| --- | --- |
| INPUT/raw/ | The plant's 18+ original workbooks / CSV / PDF files, verbatim |
| INPUT/derived/ | 36 frozen mined masters (parquet) - read directly by the engine |
| INPUT/cycletime/ | Build and cure cycle-time workbooks |
| INPUT/opening_gt_manual/ | Manually counted GT floor stock from the plant |
| schedule/send/planner/cmbc/ | THE LIVE ENGINE |
| schedule/send/warehouse/derived/ | Month-wise masters mined from MES history |
| schedule/send/runs/<name>/ | Complete output of one run - parquet plus log per layer |
| schedule/send/output/<pack>/ | Final BTP Excel pack delivered to the plant |

Important - three engines exist in the tree, only one is live planner/cmbc/ is the live engine. planner/engine/ is a superseded prototype. planner/plan/, learn/, kb/ and optimize/ are retired generation-1 code. Three separate development sessions have previously worked on the wrong codebase. Any scheduling change belongs in planner/cmbc/ only.

# 2.  Architecture - Two Phases, L0 through L12

## 2.1  The two phases

The engine is divided into two halves:

| Phase | Layers | Question it answers |
| --- | --- | --- |
| PHASE A | L0 - L4.5 | WHAT to make, and HOW MUCH? |
| PHASE B | L5 - L12 | WHERE to make it, and WHEN? |

Phase A converts demand into a net requirement and then into lots. Phase B seats those lots on actual presses and machines at actual times.

## 2.2  Full layer map

| Layer | Name | Function | Output file |
| --- | --- | --- | --- |
| L0 | Learn (offline) | Learns tau*, yields and press availability from MES history | warehouse/params/ |
| L1 | Preflight | Input gate - validates all masters, exits 1 on ERROR | preflight_<M>.parquet |
| L2 | Capability (offline) | Determines which GT can run on which machine / press / mould | cap_machine / cap_press / cap_mould / cap_ttl_groups |
| L3 | Ceiling (offline) | Derives cavities and produces the RCCP capacity report | l3_cavities.parquet |
| L4 | Net Requirement | Demand minus stock = quantity that must actually be cured | net_requirement_<M> |
| L4.5 | Lot Sizing | Splits the net requirement into CURE lots | l45_lots_<M> |
| - | Partition | Static GT-to-machine map (month-stamped) | gt_machine_partition.parquet |
| L5 | Cure Campaign Master | THE PLAN IS CREATED HERE - seats campaigns on presses | cure_campaigns.parquet |
| L7 | Pull Release | Releases building backwards (~4,000 lines - the core work) | build_schedule / gt_events / carry_forward_gt |
| L10 | Discretise | Maps time onto the shift grid and sequences the work | shift rows, mould_changes |
| L11 | Validate | Checks 40 invariants | l11_invariants.parquet |
| - | BTP Export | Produces the plant-format Excel pack | output/<pack>/ |

What happened to L6, L8, L9 and L12 These are retired (planner/cmbc/_retired/). It was verified that removing them leaves every plan artefact bit-identical and all 34 sheets of the BTP pack matching - meaning nothing was reading their output.

## 2.3  Planning a full month in one command

main.py is the orchestrator. A full August 2026 run takes approximately 30 seconds:

cd schedule/send ./.venv/Scripts/python.exe main.py plan --month 2026-08 \ --run aug_ship --out output/AUG2026_pack \ --opening-gt opening_gt_manual_2026-08.parquet

This executes the following steps. Each step's output is written to runs/<name>/log_<step>.txt:

| # | Step | What happens |
| --- | --- | --- |
| 00 | preflight | Input gate - allowable machine, PCR inch, TT/TL, press, mould, cycle time, opening GT, partition stamp |
| 00a | l2_ttl_b16 | TT/TL (tube-type / tubeless) grouping and B16 machine reach split |
| 01 | l4_net_requirement | Demand minus stock = net cure requirement |
| 02 | l45_lotsize | Creates cure lots |
| 02b | l4b_capacity_flow | GT-level and machine-level capacity flow check |
| 03 | partition | GT-to-machine static map (rebuilt if the month stamp is wrong) |
| 04 | l5_cure_master | CURE CAMPAIGNS - creation of the plan |
| 05 | l7_pull_release | BUILD SCHEDULE - backward release |
| 06 | l10_discretise | Shift grid, sequencing and mould changes |
| 07 | l11_validate_plan | 40 invariants |
| 08 | gt_sku_share | Splits GT back into SKUs for the BTP pack |
| 09 | btp_export | Plant-format Excel |
| 10 | shift_pack | 13-sheet detailed shift schedule |

# 3.  Inputs - Which Files Are Used and Why

Inputs fall into three categories:

| Type | Source | Count | Update frequency |
| --- | --- | --- | --- |
| Plant raw workbooks | Supplied by the plant team | 18+ | When the plant issues a new master |
| Frozen derived masters | Mined once from raw and frozen | 36 (INPUT/derived/) | Rarely - only when raw changes |
| MES-mined month masters | 8 months of MES history (32 M rows) | Per month | Rebuilt each month |

## 3.1  Plant raw workbooks (INPUT/raw/)

| File | What it provides | Where it is used |
| --- | --- | --- |
| August_Demand_PCR_TBR_Classification.xlsx | Monthly DEMAND, SKU-wise, PCR/TBR classified | L4 (net requirement) - the origin of everything |
| PCR BUILDING ALLOWABLE MATRIX.xlsx | Which GT is allowed on which PCR TBM machine | L7 - hard constraint |
| TBR BUILDING ALLOWABLE MATRIX.xlsx | The same for TBR (TBM 01 - TBM 09) | L7 - hard constraint |
| Recipemaster 1.xlsx + recipelookup 1.xlsx | SKU to recipe to GT chain | Building gt_sku_master |
| wcmaster 1.xlsx | Work centre master - press and machine IDs | Press / machine naming |
| mould_inv_ctp_17072026.csv | MOULD INVENTORY - how many of each mould exist | L5 - R3 constraint (concurrency limit) |
| curing_item_mould_mapping 2.csv | GT to mould mapping | cap_mould |
| Master_Mapping_Mould_SKU.csv | Mould to SKU mapping | Cross-check |
| Master_Building_ChangeoverTime_pcr/tbr.csv | Building changeover time (same size vs different size) | L7 sequencing cost |
| SKU_Construction_Clusters_PCR/TBR.xlsx | Construction clusters - which SKUs are similar | Sister-SKU grouping, changeover saving |
| percentage weightage on tbmmachine....xlsx | 1 year of machine weightage per SKU | machine_weightage - preference order |
| ALL PCR CTP SKUS.xlsx | List of all PCR CTP SKUs | Defines scope |
| CTP Set up building, curing and inspection.xlsx | Setup times | build_setup_catalogue |
| Ageing spec.pdf | GT ageing specification (basis for the 72 h limit) | Source of rule R5 |

## 3.2  Frozen derived masters (INPUT/derived/) - 36 files

These are extracted from the raw files once and then frozen. The engine reads them directly on every run rather than re-mining them. This is what allows a fresh clone to replan a full month end-to-end.

The twelve most important:

| File | Contents | Why it is critical |
| --- | --- | --- |
| allowed_machine_matrix.parquet | The plant's own statement - GT x building machine | Hard constraint. PCR eligibility median is 3 machines |
| allowed_press_matrix.parquet | GT x press eligibility (92 PCR / 79 TBR presses) | The plant's own statement; broader than the mined set |
| gt_machine_partition.parquet | Static GT-to-machine map, MONTH-STAMPED | Must be rebuilt monthly. A stale one cost July 0.58 pt |
| running_moulds_2026-08.parquet | Which mould is currently mounted on each press | L5 warm start - saves the first changeover |
| opening_gt_inventory.parquet | GT floor stock at the start of the month | Subtracted in the L4 net requirement |
| mould_cavity.parquet | Cavities per mould | Cure rate calculation |
| press_mould_change.parquet | Per-press mould change time (PCR 210-430 min) | L5 press reservation - using the median here was a defect |
| cycle_time_building.parquet | Per-machine seconds per tyre (PCR 49-78 s) | Build duration |
| cycle_time_curing.parquet | Per-press cure cycle and daily capacity | Cure duration |
| gt_size.parquet | GT rim and size | Same-size grouping, rim lock |
| tt_tl.parquet | Tube-Type vs Tubeless flag | B16 - TT and TL cannot be mixed |
| machine_rim_lock.parquet | Rim lock per machine | Rim changeovers are expensive |

The remaining 24: aging_limits, build_setup_catalogue, calendar_shifts, capacity_machine_day, capacity_press_day, gt_home_machine, gt_sister_group, gt_sku_master, lot_size, machine_gt_share, machine_weightage, pcr_inch_eligibility, prep_changeover, press_class_pcr, press_platen_master, semi_finished_ageing, sku_con_cluster, sku_construction, tbr_machine_certified, running_moulds_2026-07, gt_size.pre_rimfill, allowed_machine_matrix.bak, gt_machine_partition.provenance.json, MANIFEST.csv.

## 3.3  MES-mined month masters (warehouse/derived/)

These are derived from 8 months of MES history (December 2025 to July 2026) - approximately 32 million rows and 4.4 GB of CSV.

| File | Contents |
| --- | --- |
| net_requirement_<M>.parquet | L4 output - demand, usable stock, net_cure, gross_build |
| l45_lots_<M>.parquet | L4.5 output - lot count and lot size per GT |
| cap_machine_<M> / cap_press_<M> | Mined eligibility - what actually ran in the MES window |
| cap_mould_<M> | Mould count and max_concurrent_presses (the basis for R3) |
| cap_ttl_groups_<M> | TT/TL group split |
| l3_cavities.parquet | Per-press effective cavities and cycle_s |
| b16_machine_reach_<M> | B16 gate - which GTs a machine can reach |

An important nuance - there are two separate path resolvers input_derived() and wh_derived() are deliberately separate functions. 21 filenames exist in both trees; twenty are byte-identical, but press_mould_change.parquet is not. L5 and L10 read the warehouse copy. Merging the two resolvers would silently change the cure schedule.

## 3.4  Ingesting a new month

When the plant supplies new files, these two commands are run:

# Demand $PY -m scripts.ingest_orderbook_demand \ --xlsx "../../INPUT/raw/August_Demand_PCR_TBR_Classification.xlsx" \ --sheet "August Demand Classified" --month 2026-08 # Opening GT (manual floor count) $PY -m scripts.ingest_manual_opening_gt --month 2026-08 --age-h 24 \ --pcr "../../INPUT/opening_gt_manual/gt_inventory_manual_pcr_20260801.xlsx" \ --tbr "../../INPUT/opening_gt_manual/gt_inventory_manual_tbr_20260801.xlsx"

# 4.  End-to-End Flow, Step by Step

This section traces the complete path: demand arrives, is split into PCR and TBR, the allowable check is applied, and planning and curing follow.

## 4.1  STEP 1 - Demand arrives and is split into PCR / TBR

The plant demand workbook (August_Demand_PCR_TBR_Classification.xlsx) provides SKU-level demand, already classified as PCR or TBR. The ingest script then:

Translates SKU to GT code via gt_sku_master, using the curing-recipe chain.

Separates PCR from TBR using the plant column.

Aggregates multiple SKUs that map to the same GT.

August 2026 result:

| Plant | GT codes | Demand (tyres) |
| --- | --- | --- |
| PCR | 73 | 4,29,146 |
| TBR | 37 | 99,019 |
| TOTAL | 110 | 5,28,165 |

The GT namespace trap - this has caused debugging effort twice The plant writes GT codes in at least four different formats. The engine plans in exactly one (v_build.itemCode). The TBR BOM writes "GT 5001" while TBR MES uses a size-led format - the two namespaces have ZERO string overlap. The bridge is therefore always made on SKU, never on GT code. scripts/gt_namespace.py is the single bridge; it returns None on ambiguity rather than guessing.

## 4.2  STEP 2 - Preflight (L1): the input gate

Before any planning begins, the engine validates all masters. This gate requires no MES data:

Is the allowable machine matrix present, and does it cover every demand GT?

Are PCR inch eligibility and TT/TL groups correct?

Are the press master, mould master and cycle times present?

Is the opening GT file available?

IS THE PARTITION MONTH STAMP CORRECT? If not, this is a blocking ERROR and the run exits 1.

Output: preflight_<M>.parquet. If any ERROR is raised the engine does not proceed. This is deliberate - no plan is preferable to a plan built on an incorrect master.

## 4.3  STEP 3 - L4: Net Requirement

The calculation is direct:

net_cure = demand - usable_opening_stock gross_build = ceil(net_cure / cure_yield) (R15)

Cure yield is learned from MES history: PCR 0.99712 (n = 29,95,222) and TBR 0.98202 (n = 7,53,263). Build yield is deliberately left as null - the MES QualityStatus column has only one distinct value, so no build scrap signal exists and 1.0 was not assumed.

August 2026:

| Plant | Demand | Usable stock | From stock | Net cure | Gross build |
| --- | --- | --- | --- | --- | --- |
| PCR | 4,29,146 | 4,418 | 4,395 | 4,29,146 | 4,26,028 |
| TBR | 99,019 | 1,028 | 1,028 | 99,019 | 99,822 |

Note that opening stock is very low - PCR has only 4,418 tyres, equivalent to roughly 8 hours of production. There is therefore no cushion at the start of the month, which is a significant contributor to the start-of-month dip discussed in Section 7.

## 4.4  STEP 4 - L4.5: Lot Sizing

The net requirement is split into CURE lots. Note that this is the cure lot; build slices have no minimum.

min_cure_lot = moulds x cavities x (campaign_min_h x 60 / cycle_min)

| Plant | GT count | Need | Lots created | min_lot (p50) | B12 fixed floor |
| --- | --- | --- | --- | --- | --- |
| PCR | 73 | 4,26,028 | 389 | 202.8 | 150 |
| TBR | 37 | 99,822 | 221 | 70.0 | 70 |

The B12 lot floor - the principal TBR constraint The TBR min_lot is fixed at 70 across all 37 GTs. When a GT's remaining requirement falls below 70 it cannot be built - recorded as "would breach min_lot". Of TBR's 6,330 unfed tyres, 6,100 (96 %) arise from this. This is not an engineering problem; it requires a PLANT RULING on whether sub-floor lots are permitted.

## 4.5  STEP 5 - Allowable check and Partition

These are two distinct concepts and are frequently confused:

|  | ALLOWABLE MATRIX | PARTITION |
| --- | --- | --- |
| What it is | A plant rule - is GT x machine permitted or not | An engine decision - which machine the GT is assigned to |
| Source | The plant's workbook | CP-SAT solver, from demand plus capacity |
| Nature | Hard constraint (yes / no) | Assignment / preference |
| Validity | Until the plant revises it | One month only - month-stamped |

Semantics of the allowable check:

If (plant, GT) appears in the matrix, ONLY the listed machines are permitted; all others are dropped.

If (plant, GT) is absent, the mined eligibility applies unchanged, so that a new GT never becomes unplannable.

The partition must be rebuilt every month The partition is sized against one month's demand and calendar hours. L7 REFUSES to plan on a partition stamped for a different month. In July a stale partition was used: it cost 0.58 pt of fulfilment and 10.3 pt of same-size share, and every gate passed. It is now a blocking ERROR.

## 4.6  STEP 6 - L5: Cure Campaign Master (creation of the plan)

This is the most important layer - the plan is created here.

L5 performs the following:

Places all cure lots in a queue, largest first (scarcity priority).

Finds eligible presses for each lot (cap_press plus allowed_press_matrix).

Selects the press that becomes free earliest.

Applies the R3 check - concurrent presses for a GT cannot exceed its mould count.

Reserves mould change time, per press (210-430 minutes on PCR).

Fixes the campaign start_ts and end_ts.

One critical mechanism - the RELEASE FLOOR:

PCR release floor = tau* + build_band = 4.319 h + 7.54 h = 11.86 hours TBR release floor = 4.809 h + 5.37 h = approximately 10.18 hours

This means no press can start immediately at t0 (1 August 07:00). Every press must wait at least 11.86 hours on PCR, because building must first produce GT. This is a second major contributor to the start-of-month dip.

August 2026 L5 output:

| Plant | Campaigns | Quantity | Press hours | Presses used |
| --- | --- | --- | --- | --- |
| PCR | 280 | 4,12,897 | 65,677 | 86 |
| TBR | 191 | 98,625 | 58,989 | 79 |

## 4.7  STEP 7 - L7: Pull Release of Building (~4,000 lines)

Building is now released backwards. For each cure campaign:

build_start = cure_start - tau* - build_duration

L7 is responsible for:

Finding an eligible building machine for each campaign, within the allowable matrix.

Finding a slot on that machine, adjusting earlier or later where it is occupied.

Applying the B16 gate so that TT and TL are not mixed.

Minimising changeover cost by keeping same-size and sister SKUs together.

Maintaining the GT ledger - +1 per tyre built, -1 per tyre cured.

Recording anything that cannot be fed into build_starved.parquet.

One important rule inside L7: when a machine is busy, build EARLIER, never later. Moving a slice later starves the press, and a lost press-hour is unrecoverable. Moving it earlier only ages the tyre, and it has 72 hours of shelf life available.

August 2026 L7 output:

| Plant | Build rows | Built qty | Machines | Utilisation | Starved |
| --- | --- | --- | --- | --- | --- |
| PCR | 2,635 | 4,08,950 | 11 | 83.5 % | 2,066 |
| TBR | 3,676 | 97,985 | 9 | 81.7 % | 27 |

Building is not late - this has been measured Build-to-cure wait: PCR p50 = 4.3 hours, TBR p50 = 4.7 hours. Maximum 71.6 h and 68.6 h - the 72-hour limit is never breached. Only 0.5 % of PCR tyres record "no feasible release". Of the presses that started late, only 4 on PCR and 7 on TBR were waiting on building; for the remaining 68 PCR and 62 TBR presses the GT was already available on the floor.

## 4.8  STEP 8 - L10: Discretise and Sequence

Continuous time is mapped onto the plant shift grid:

The plant day runs 07:00 to 07:00, not from wall-clock midnight.

Shift A = 07-15, Shift B = 15-23, Shift C = 23-07.

The "date" in exports is the PLANT-DAY date, not the wall-clock date. Labelling C shift by wall clock once mislabelled 28.7 % of build rows.

Mould changes are listed explicitly in mould_changes.parquet.

## 4.9  STEP 9 - L11: Validation (40 invariants)

The plan is checked against 40 separate rules. Examples include:

No machine is running two lots at the same time (overlap check).

No mould is double-booked.

The GT ledger never goes negative (starvation check).

R5 - no GT waits longer than 72 hours.

G8 - GT WIP stays within the rail (PCR 4,800 / TBR 1,400).

Same-size share remains above threshold.

Sub-floor run share does not exceed the plant's own share.

## 4.10  STEP 10 - BTP Export and Verification

Finally the plant-format Excel pack is produced and an independent verifier is run:

$PY scripts/export_shift_schedule.py aug_ship 2026-08 output/AUG2026_pack $PY scripts/verify_export.py output/AUG2026_pack 2026-08

The verifier deliberately does not import the planner verify_export.py and validate/violations.py re-derive every check from the exported files. If the verifier called planner internals it would only prove that the planner agrees with itself. This independence is intentional.

# 5.  Business Rules That Govern the Plan

There are 46 numbered rules in total (documented in BUSINESS_RULES.md with B/P/C/S/G/E prefixes). Those with the greatest impact are:

| Rule | Statement | Value | Effect |
| --- | --- | --- | --- |
| R3 | Concurrent presses for a GT <= its mould count | Per GT | Limits parallel capacity for large GTs |
| R5 | GT shelf life | 72 hours | Prevents building too early. Not env-overridable |
| B12 | Build lot floor | PCR 150 / TBR 70 | Small residual quantities cannot be built |
| G8 | GT WIP rail | PCR 4,800 / TBR 1,400 | Prevents GT accumulation |
| B16 | TT / TL separation | Group-wise | Tube-type and tubeless are not mixed |
| B7 | Daily build quota | REJECTED | Trialled; our interior CV was already better than the plant's |

Caps are defined in exactly one place - config.py The governing principle is "add a cap in config.py or nowhere". L7 and L11 both read from there and neither keeps its own copy. A duplicated value once became a defect: L7 enforced 1,400 while L11 graded against 1,500.

## 5.1  Key learned parameters (from L0)

| Parameter | PCR | TBR | Meaning |
| --- | --- | --- | --- |
| tau* (coupling buffer) | 4.319 h | 4.809 h | Median gap between build and cure |
| tau_min | 0.268 h | 0.268 h | Physically shortest possible gap |
| build_band (p50) | 7.54 h | ~5.37 h | Median campaign length |
| Press availability | 0.8897 | 0.8282 | Derived from MTBF / MTTR |
| Mould change time | 210-430 min (median 360) | 361 min (flat) | Per press |
| Cure yield | 0.99712 | 0.98202 | Learned from MES; build yield deliberately null |

The most important lesson - a mined statistic is not a constraint Both tau* and min_lot were plant MEDIANS that were wired in as hard floors. Together they cost 13.4 POINTS of fulfilment. A median means that 50 % of reality already performs better than it. The diagnostic sign: if p01, p05 and p10 are all identical, a wall has been constructed rather than a distribution observed.

# 6.  August 2026 - Actual Results

## 6.1  Volume waterfall

| Stage | PCR | TBR | What happens here |
| --- | --- | --- | --- |
| Demand | 4,29,146 | 99,019 | Plant order book |
| Net cure requirement (L4) | 4,29,146 | 99,019 | Opening stock was very low |
| Cure campaigns placed (L5) | 4,12,897 | 98,625 | Press capacity full - 16,249 / 394 not placed |
| BUILT (L7) | 4,08,950 | 97,985 | 2,066 / 27 starved |
| Fed in-month | 3,84,464 | 92,983 | Remainder cures after the month boundary (tail) |
| Carry out to September | ~24,486 | ~5,002 | This is output, not loss |

## 6.2  Capacity picture - identifying the bottleneck

| Resource | PCR | TBR |
| --- | --- | --- |
| Building machines | 11 | 9 |
| Build capacity (nameplate) | 5,02,839 (117 % of demand) | 1,17,572 (119 %) |
| Build utilisation | 83.5 % | 81.7 % |
| Build idle capacity | 86,352 tyres | 21,686 tyres |
| Presses used | 86 | 79 |
| Cure ceiling (as modelled) | 4,07,396 (95 % of demand) | 1,06,415 (107 %) |
| Press utilisation | 102.6 % (extends past month end) | 100.4 % |

The conclusion is unambiguous PCR: the cure ceiling is 4,07,396 against demand of 4,29,146, meaning the plan is 5 % short before any scheduling decision is taken. Building has 86,352 tyres of spare capacity, but this cannot be used because curing cannot absorb the output. TBR: the cure ceiling is 107 % of demand, so TBR is NOT capacity-bound. Its constraint is the B12 lot floor.

## 6.3  Per-machine building load (PCR) - imbalance

This is a genuine issue. It is not currently the bottleneck, but it will become one as soon as cure capacity is increased:

| Machine | sec/tyre | Capacity/month | Built | Utilisation | Idle capacity |
| --- | --- | --- | --- | --- | --- |
| TBMPCR5 | 49 (fastest) | 54,661 | 27,686 | 52.9 % | 25,741 |
| TBMPCR7 | 49 (fastest) | 54,661 | 40,555 | 75.8 % | 13,222 |
| TBMPCR4 | 49 | 54,661 | 45,535 | 91.9 % | 4,423 |
| TBMPCR10 | 51 | 52,518 | 51,544 | 94.0 % | 3,157 |
| TBMPCR8 | 66 | 40,582 | 37,589 | 94.5 % | 2,236 |
| TBMPCR11 | 69 (slow) | 38,817 | 34,692 | 97.0 % | 1,151 |
| TBMPCR9 | 78 (slowest) | 34,338 | 31,866 | 86.9 % | 4,503 |

Note that the fastest machine (TBMPCR5, 49 s/tyre) runs at only 52.9 %, while the slowest machines run at 87-97 %. The reason is that building is released backwards from the cure campaign: machine selection follows whichever campaign requires feeding, not machine speed.

# 7.  Start-of-Month and End-of-Month Dip - Full Explanation

## 7.1  The data

| Day | Cure PCR | Cure TBR | Press util PCR | Press util TBR | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | 4,386 | 1,085 | 33.0 % | 34.1 % | START DIP - severe |
| 2 | 10,352 | 2,372 | 77.0 % | 74.7 % | START DIP - recovering |
| 3 | 12,709 | 3,028 | 95.6 % | 95.6 % | Almost normal |
| 4 | 13,276 | 3,082 | 99.7 % | 97.4 % | NORMAL |
| 5 - 15 | ~13,300 | ~3,085 | ~100 % | ~97.5 % | STEADY STATE |
| 16 - 28 | ~12,500 | ~3,120 | ~98 % | ~98 % | Steady, slightly lower |
| 29 | 12,566 | 3,071 | 97.0 % | 96.7 % | END DIP begins |
| 30 | 11,960 | 3,051 | 94.3 % | 96.2 % | END DIP |
| 31 | 11,583 | 2,912 | 92.4 % | 92.0 % | END DIP - lowest |

## 7.2  START DIP - three causes acting together

CAUSE 1: Cold start - there is no carry-in (the largest factor)

At the start of the month the engine assumes nothing is running. In the real plant, machines are running on the night of 31 July and continue running on the morning of 1 August. Our plan starts everything from zero at t0 (1 August 07:00). This loses the first 12 to 24 hours of production window.

CAUSE 2: The release floor wall - 11.86 hours on PCR

L5 holds every press for tau* + build_band = 11.86 hours. The logic is that building must first produce GT. However both terms are MEDIANS - tau* is the plant's median coupling buffer (47 % of tyres cure sooner than this) and build_band is the median campaign LENGTH, not the time to build the first lot. Physically the first legal lot can be produced in 2.35 hours on PCR, not 11.86.

CAUSE 3: Opening GT stock is very low

PCR had only 4,418 usable GT (approximately 8 hours of curing) and TBR only 1,028. With a larger opening stock the presses could start immediately, giving building time to catch up.

The critical finding - building was not late On day 1, only 4 of 86 PCR presses and 7 of 79 TBR presses were waiting on building. The remaining 68 PCR presses (1,888 hours) and 62 TBR presses (2,368 hours) were idle even though their GT was already available on the floor. Example: GT 1513 XPC1 MSIL was built by hour 2.2, but its 11 presses did not start until hour 24.4 - 244 press-hours wasted. The press was waiting on a CLOCK, not on STOCK.

## 7.3  END DIP - three causes

CAUSE 1: The horizon boundary

A campaign that would extend beyond 31 August does not count as in-month. L5 is therefore reluctant to seat such campaigns, so less work is available in the final days. 56 campaigns cross the 1 September boundary, carrying 27,118 tyres.

CAUSE 2: L4.5 freezes quantities in advance

Lot quantities are fixed in L4.5, before any press is seated. Consequently, when a press becomes free at the end of the month, no campaign can GROW to absorb it. The plant does exactly this; our engine cannot.

CAUSE 3: We front-load where the plant back-loads

Measured on GT 1513 (the largest PCR GT: 55,663 tyres, 35 moulds, 44 eligible presses):

|  | Days 1-22 | Days 23-31 | Pattern |
| --- | --- | --- | --- |
| Plant | 33,415 | 22,248 | 2,472/day - ramps 60 % on day 23 |
| Our plan | 38,998 | 15,103 | 1,678/day - decays steadily |

The plant uses its large, high-mould GTs as an ABSORBER. As smaller GTs finish and presses become free, the plant re-tools them onto GT 1513, which has mould headroom to take them. This is why the plant has no month-end idle time while our plan leaves 1,085 press-hours idle in the final five days.

## 7.4  Full dip calculation - and why it matters

Using the steady-state daily rate to compute the loss:

| Dip | PCR loss (tyres) | TBR loss (tyres) | Calculation |
| --- | --- | --- | --- |
| Day 1 | 8,931 | 2,000 | Steady 13,317 minus actual 4,386 |
| Day 2 | 2,965 | 713 | 13,317 minus 10,352 |
| Day 3 | 608 | 57 | 13,317 minus 12,709 |
| START DIP TOTAL | 12,504 | 2,770 |  |
| Day 29 | 751 | 69 | ~13,317 minus 12,566 |
| Day 30 | 1,357 | 89 |  |
| Day 31 | 1,734 | 228 |  |
| END DIP TOTAL | 3,842 | 386 |  |
| GRAND TOTAL | 16,346 | 3,156 |  |

This is the single most important insight in this document The total PCR gap (demand 4,29,146 minus campaigns placed 4,12,897) is 16,249 tyres. The total dip loss is 16,346 tyres. These are almost exactly equal. In other words, the entire PCR shortfall consists of the start dip and the end dip. Across the middle 25 days the presses run at 98-100 %, where there is no problem to solve. Correcting both dips would bring PCR demand close to fully met.

## 7.5  Options for correcting the dip

| Solution | Action required | Benefit | Decision owner |
| --- | --- | --- | --- |
| Warm start / carry-in | Take the running state of 31 July night as an input | Most of day 1 | PLANT RULING - the ~4 h carry-in question |
| Correct the release floor | Replace tau* + band with per-GT actual build time | ~1,090 PCR + 1,496 TBR press-hours | ENGINEERING (l56_loop) |
| Increase opening GT | Leave slightly more GT at month end | Day 1 cushion | PLANT RULING |
| Phased order book | Request weekly due dates in the demand file | Corrects the flat curve, reduces end dip | PLANT - data |
| Back-loading (absorber) | Reserve large mould-rich GTs for month end | End dip | ENGINEERING (PLANNER_L5_BACKLOAD) |
| Correct the press roster | Restore PCR presses 17-22 | ~24,552 tyres of ceiling | PLANT RULING - are they decommissioned? |

An important related point The demand file contains no phasing - all 5,28,165 August tyres carry a single date (day 31). The engine therefore has no deadlines and simply sorts by size, placing work at the earliest feasible time. This produces a FLAT concurrency curve. If the plant can supply weekly due dates this may correct itself; the flag already exists in the code (PLANNER_L5_EDD) and only the data is missing.

# 8.  Known Gaps

This section is deliberately candid. Each issue is marked as either an ENGINEERING task or a PLANT RULING.

## 8.1  In priority order

| # | Issue | Impact | Type |
| --- | --- | --- | --- |
| 1 | PCR presses 17-22 (6 presses) are invisible to the planner | ~24,552 tyres/month of cure ceiling | PLANT RULING |
| 2 | Demand has no phasing (all dated day 31) | Flat curve, end dip | PLANT - data |
| 3 | The release floor is median-based (11.86 h) | ~2,586 press-hours = ~9,400 tyres | ENGINEERING |
| 4 | TBR B12 lot floor | 6,100 of 6,330 unfed TBR tyres | PLANT RULING |
| 5 | Cure capacity is modelled from achieved history | Ceiling 95 % vs plant workbook 106 % | ENGINEERING + PLANT |
| 6 | Building machine imbalance (TBMPCR5 at 52.9 %) | Not yet binding; will bind later | ENGINEERING |
| 7 | gt_size rim coverage - 28 GTs have no rim | Weak rim grouping | PLANT - data |
| 8 | Carry-in / horizon boundary question | +1.0 to 1.8 pt per plant | PLANT RULING |

## 8.2  Issue 1 in detail - how six presses are missing

masters/press_list_<month>.json is a hand-maintained file, and its press count has been declining:

| Month | PCR presses | Includes 17-22? |
| --- | --- | --- |
| 2026-05 | 91 | Yes - 20, 21, 22 |
| 2026-06 | 89 | Only 21, 22 |
| 2026-07 | 86 | No |
| 2026-08 | 86 | No |

These presses are real: MES records them curing 138-142 tyres per day, and the plant's own allowed_press_matrix marks them as "direct" for 5 GTs carrying 35,332 tyres of August demand. GT 1772 NEO (1,178 tyres) is planned to zero because press 22 is its only permitted press.

This file has no generator press_list_<month>.json is not produced by any script, has no provenance record, and appears in no defect ledger. A hand-edited JSON file is silently setting the plant's cure ceiling. The question for the plant is whether presses 17-22 were decommissioned, or were simply idle during the mining window.

## 8.3  Deliberate non-goals - these should not be "fixed"

No ML / RL / LLM - the plant requires explainable output.

No CP-SAT / MILP in the scheduler - trialled and measured, no benefit found.

Daily build quota (B7) REJECTED - our interior CV is already better than the plant's.

GT inventory sitting below the G8 band is INTENTIONAL - GT is a buffer, not a target.

The curing cycleStart field is named backwards (press-open = cycle END). This reflects the source data and must not be "corrected".

Moulds are held per (plant, GT, press). Forcing a single primary mould per GT produced 4.16 lakh phantom violations.

# 9.  Glossary

| Term | Definition |
| --- | --- |
| GT | Green Tyre - uncured tyre, after building and before curing. 72-hour shelf life |
| PCR | Passenger Car Radial |
| TBR | Truck Bus Radial |
| TBM | Tyre Building Machine |
| Press | Curing press - where the GT is cured |
| Mould | The forming tool fitted inside a press. Held per (plant, GT, press) |
| Cavity | Number of tyres a press cures simultaneously - PCR ~3.4 effective, TBR ~2.4 |
| tau* (tau star) | Median gap between build and cure. PCR 4.319 h, TBR 4.809 h |
| Campaign | A continuous run of one GT on one press |
| Lot | A batch. The cure lot has a floor; the build slice does not |
| Changeover | Reconfiguring a machine or press for a different GT - consumes time |
| Same-size share | Proportion of production run consecutively in the same size - higher means fewer changeovers |
| BUILT | Tyres actually produced this month (excluding the OPENING_STOCK pseudo-machine) |
| Fed | Tyres delivered into presses - includes opening stock |
| Cured in-month | The fulfilment numerator = built + opening - closing |
| Carry-out / tail | Campaigns continuing past the month boundary - this is output, not loss |
| BTP | The plant's target Excel format for the final schedule |
| MES | Manufacturing Execution System - the plant's actual production data |
| Invariant | A condition that must always hold - L11 checks 40 of them |
| R3 | Concurrent presses <= mould count |
| R5 | GT shelf life of 72 hours |
| B12 | Build lot floor - PCR 150, TBR 70 |
| G8 | GT WIP rail - PCR 4,800, TBR 1,400 |
| B16 | Tube-Type and Tubeless must be kept separate |

--- End of Document ---

All figures are derived from the actual artefacts of runs/FINAL_aug (month 2026-08) and can be independently re-verified.